Title: 	Clarification of policy permitting the Consolidation of Tasks,

Submitter:	Cliff Neudorf (TCAA) and Brent Bringle (FAA)

Issue:	OEMs are combining multiple tasks during the task consolidation phase of the MSG-3 analysis process. Neither the MSG-3 document nor the IMRBPB have established good guidance on what tasks may be combined and what may not be combined. With reference to Section 2-3-7 of the MSG-3 document it is worrisome, that while this section requires the identification and development of tasks to detect the degradation displayed by a particular failure cause, such tasks are then being combined just because they might have the same accomplishment interval and they might be performed in the same zone.  This is occurring; despite the fact these tasks may be developed to address failure causes with vastly different forms of degradation.

Problem: 	During the completion phase of the MSG-3 Level 2 analysis for an MSI, analysts will consolidate into a single task, tasks which are designed to find widely varying failure causes.

With reference to MSG-3 Section 2-3-7 Task Development (Second Level) such tasks are:

- Lubrication / Servicing tasks(applicable to all failure effect categories)

- Operational /Visual Check tasks(applicable to hidden functional failures only)

- Inspection (GVI,DET,SDI) / Functional Check tasks(applicable to all categories)

- Restoration tasks(applicable to all FE categories)

- Discard tasks(applicable to all FE categories)

- Combination of tasks (applicable to safety category tasks only)

The consolidation process practiced by the OEMs will at times combine failure finding tasks such as Operational Checks and Visual Checks with each other or with potential failure finding Inspection tasks or Functional Checks and may even mix any of these tasks with failure avoidance Lubrication or Servicing tasks.

The following is but one example of such a consolidation exercise:

MSI 24-24 (Task Description)

FUNCTIONAL AND OPERATIONAL CHECK OF THE RAT, DETAILED VISUAL INSPECTION OF RAT MECHANICAL MECHANISM, OIL LEVEL VISUAL CHECK AND LUBRICATION OF STRUCTURAL ATTACHMENTS.

In examining current PPHs, it appears that this problem stems from the fact that we Regulatory Authorities have permitted the OEMs to include within their Policy and Procedures Manuals guidance that permits the OEM to perform these types of consolidation.  Please see the attached excerpts from current PPHs used on recently type certified aircraft. It seems that we as Regulatory Authorities must take harmonized action to ensure that the PPHs contain more stringent criteria to guide the OEM and the Analysts when combining or consolidating tasks.

Recommendation (including Implementation):

It is recommended that policy be established by the regulatory authorities as follows:

- That for all new MRB processes, the corresponding PPH must contain guidance on consolidation or combining of tasks which meets the following points, and

- That for existing MRB reports which will or may undergo an evolution exercise that the evolution exercise may only be accomplished when  the corresponding PPH is amended to include guidance that existing consolidated tasks in the MRBR will be reviewed and returned to their original state if they do not meet the following criteria:

  - That Consolidation of failure preventative tasks such as Lubrication or Servicing Tasks with failure finding tasks such as Operational Checks, and Visual Checks may not be approved in MRBRs and that none of the above  tasks may be consolidated with potential failure finding tasks such as GVI, DET, SDI, Zonal Inspections or Functional Checks, and

  - That potential failure finding tasks such as GVI, DET, SDI, FNC may not be consolidated with each other, and

  - That existing MRBRs where this consolidation may have mistakenly been permitted shall be reopened and the consolidated tasks returned to their original state in order to mitigate any potential safety issues.

IMRBPB Position:

Date:

Position:

Status of Issue Paper (when closed state the closure date):

Recommendation for implementation:

Important Note:  The IMRBPB positions are not policy.  Positions become policy only when the policy is issued formally by the appropriate National Aviation Authority.

.